Demo Story (1-liner)

A sudden traffic surge causes congestion on an aggregation link, leading to queue drops, latency spikes, and application slowness. The system must correlate metrics, events, topology, and time to form a single situation and identify the congested link as the root cause.

1) Demo Scope (Correlation Only)

Goal: From multi-source telemetry, form one correlated situation around Link Congestion on Agg-SW1 and correctly group symptoms (Edge + App) using:

- Temporal correlation

- Spatial correlation

- Topological correlation

- Causal / Rule-based correlation

- (Optionally) ML/GNN refinement

- LLM used for semantic correlation synthesis (not RCA)

2) Detailed Dataset

2.1 Topology (static)

Users

|

Edge-R3 ----\

\

>---- Agg-SW1 ---- Core-R1 ---- DC Services

/

Edge-R4 ----/

Adjacency list

- Edge-R3 → Agg-SW1

- Edge-R4 → Agg-SW1

- Agg-SW1 → Core-R1

Metadata

- Site: BLR-DC

- Zone: Z1

- Roles: Edge, Aggregation, Core, App-Gateway

2.2 Events (Alarms)

| event_id | timestamp | device | event_code | severity | site | zone | text |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E001 | 2025-01-01 10:00:05 | Agg-SW1 | LINK_UTIL_HIGH | MAJOR | BLR-DC | Z1 | Uplink utilization high |
| E002 | 2025-01-01 10:00:10 | Agg-SW1 | QUEUE_DROP | MAJOR | BLR-DC | Z1 | Queue drops detected |
| E003 | 2025-01-01 10:00:20 | Edge-R3 | LATENCY_HIGH | MINOR | BLR-DC | Z1 | Latency spike |
| E004 | 2025-01-01 10:00:25 | Edge-R4 | LATENCY_HIGH | MINOR | BLR-DC | Z1 | Latency spike |
| E005 | 2025-01-01 10:00:40 | App-GW1 | RESPONSE_TIME_HIGH | MINOR | BLR-DC | Z1 | App response time degraded |

2.3 Metrics (TSDB excerpts)

| time | device | metric | value |
| --- | --- | --- | --- |
| 09:59:30 | Agg-SW1 | if_util_percent | 62 |
| 10:00:00 | Agg-SW1 | if_util_percent | 91 |
| 10:00:10 | Agg-SW1 | if_util_percent | 95 |
| 10:00:10 | Agg-SW1 | queue_depth | 88% |
| 10:00:15 | Agg-SW1 | packet_drop_rate | 3.2% |
| 10:00:20 | Edge-R3 | rtt_ms | 180 |
| 10:00:25 | Edge-R4 | rtt_ms | 175 |
| 10:00:40 | App-GW1 | p95_response_time_ms | 2400 |

Baselines:

- Normal if_util: < 70%

- Normal RTT: < 60 ms

- Normal p95 app RT: < 500 ms

2.4 NetFlow (summarized)

| time window | link | top_talker_subnet | bps_gbps |
| --- | --- | --- | --- |
| 09:55–10:05 | Agg-SW1→Core-R1 | 10.20.0.0/16 | 8.5 |
| capacity | Agg-SW1→Core-R1 | — | 10.0 |

Observation: sudden surge from 10.20.0.0/16 pushing link near capacity.

2.5 Configuration / Change Log (context signal)

| time | device | change_id | summary |
| --- | --- | --- | --- |
| 09:50:00 | Agg-SW1 | C-7781 | QoS policy updated |

(Used only as a supporting signal for correlation strength; not for RCA.)

3) Correlation Engine — Step-by-Step Logic

3.1 Preprocessing & Enrichment

Inputs: Events, Metrics, NetFlow, Topology, Changes
Actions:

- Normalize timestamps to UTC

- Enrich each event with:

  - site=BLR-DC, zone=Z1, role (Edge/Agg/Core/App)

  - upstream/downstream neighbors from topology

- Build Event Graph Nodes:

  - Nodes: E001..E005

  - Attach metric snapshots near each timestamp

3.2 Temporal Correlation

Rule: If two events occur within Δt ≤ 60 seconds, create a temporal edge with score S_t = 1 - (Δt / 60).

Compute pairwise:

- E001 (10:00:05) ↔ E002 (10:00:10): Δt=5s → S_t=0.92

- E001 ↔ E003: Δt=15s → S_t=0.75

- E002 ↔ E003: Δt=10s → S_t=0.83

- E003 ↔ E004: Δt=5s → S_t=0.92

- E004 ↔ E005: Δt=15s → S_t=0.75

Result: All events form a temporal cluster window around 10:00.

3.3 Spatial Correlation

Rule: Same site + zone → spatial edge with S_s = 0.6.
(All events are BLR-DC, Z1.)

Edges:

- E001 ↔ E002, E003, E004, E005: S_s=0.6

- E003 ↔ E004 ↔ E005: S_s=0.6

Result: Confirms locality (not a distributed/global issue).

3.4 Topological Correlation

Rule: If event A’s device is upstream of event B’s device in topology, create topological edge with score based on hop distance:

- 0 hops (same device): 1.0

- 1 hop: 0.8

- 2 hops: 0.6

Topology distances:

- Agg-SW1 → Edge-R3: 1 hop → 0.8

- Agg-SW1 → Edge-R4: 1 hop → 0.8

- Agg-SW1 → App-GW1: 2 hops (via Core-R1) → 0.6

Edges:

- E001/E002 (Agg-SW1) → E003 (Edge-R3): S_topo=0.8

- E001/E002 → E004 (Edge-R4): S_topo=0.8

- E001/E002 → E005 (App-GW1): S_topo=0.6

Result: Shows impact propagation path.

3.5 Causal / Rule-Based Correlation

Rules (domain):

- LINK_UTIL_HIGH → QUEUE_DROP (on same device) is a causal chain (score 0.9)

- QUEUE_DROP upstream → LATENCY_HIGH downstream is causal (score 0.8)

- LATENCY_HIGH → RESPONSE_TIME_HIGH is causal (score 0.7)

Apply:

- E001 → E002 (Agg-SW1): causal, 0.9

- E002 → E003 (Edge-R3): causal, 0.8

- E002 → E004 (Edge-R4): causal, 0.8

- E003/E004 → E005 (App-GW1): causal, 0.7

Result: Produces a directed causal chain.

3.6 Candidate Incident Formation (Rule Fusion)

For each event pair, compute combined correlation score:

S_total = w_t * S_t + w_s * S_s + w_topo * S_topo + w_c * S_causal

Example weights: w_t=0.25, w_s=0.15, w_topo=0.30, w_c=0.30

Examples:

- E001→E002:

  - S_t=0.92, S_s=0.6, S_topo=1.0 (same device), S_causal=0.9

  - S_total ≈ 0.250.92 + 0.150.6 + 0.301.0 + 0.300.9 ≈ 0.90

- E002→E003:

  - S_t=0.83, S_s=0.6, S_topo=0.8, S_causal=0.8

  - S_total ≈ 0.81

- E002→E005:

  - S_t≈0.58, S_s=0.6, S_topo=0.6, S_causal=0 (no direct)

  - S_total ≈ 0.48 (weak but still related)

Thresholds:

- ≥ 0.7 → strong link

- 0.4–0.7 → weak link

- < 0.4 → ignore

Resulting graph: One connected component with:

- Strong core around Agg-SW1 (E001, E002)

- Downstream symptoms (E003, E004, E005)

This becomes one correlated situation candidate.

3.7 (Optional) GNN Refinement

Input graph:

- Nodes: E001..E005 with features:

  - device role, event type, severity, metric deltas

- Edges: temporal, spatial, topo, causal with weights

GNN objective:

- Learn node influence scores and edge importance

- Output refined weights:

  - E001, E002 high centrality

  - E003, E004 medium

  - E005 lower (leaf impact)

Output (example):

- Node importance:

  - E002 (QUEUE_DROP Agg-SW1): 0.92

  - E001 (LINK_UTIL_HIGH Agg-SW1): 0.88

  - E003/E004: ~0.45

  - E005: ~0.30

Used to rank nodes inside the situation (still correlation-only).

4) LLM for Correlation — Exact Input & Output

4.1 What the LLM Receives (Exact Payload)

System instruction (short):

You are a correlation reasoning assistant. Given events, metrics, topology, and precomputed correlations, summarize the correlated situation and explain which events belong together and why. Do NOT perform RCA or suggest fixes.

User payload (structured):

{

"topology": {

"edges": [

["Edge-R3", "Agg-SW1"],

["Edge-R4", "Agg-SW1"],

["Agg-SW1", "Core-R1"]

]

},

"events": [

{"id":"E001","time":"10:00:05","device":"Agg-SW1","code":"LINK_UTIL_HIGH"},

{"id":"E002","time":"10:00:10","device":"Agg-SW1","code":"QUEUE_DROP"},

{"id":"E003","time":"10:00:20","device":"Edge-R3","code":"LATENCY_HIGH"},

{"id":"E004","time":"10:00:25","device":"Edge-R4","code":"LATENCY_HIGH"},

{"id":"E005","time":"10:00:40","device":"App-GW1","code":"RESPONSE_TIME_HIGH"}

],

"metrics_summary": {

"Agg-SW1": ["if_util=95%", "queue_depth=88%", "drop_rate=3.2%"],

"Edge-R3": ["rtt=180ms"],

"Edge-R4": ["rtt=175ms"],

"App-GW1": ["p95=2400ms"]

},

"correlation_edges": [

{"from":"E001","to":"E002","type":"causal","score":0.9},

{"from":"E002","to":"E003","type":"topological","score":0.8},

{"from":"E002","to":"E004","type":"topological","score":0.8},

{"from":"E003","to":"E005","type":"causal","score":0.7},

{"from":"E001","to":"E003","type":"temporal","score":0.75},

{"from":"E003","to":"E004","type":"temporal","score":0.92}

],

"gnn_node_scores": {

"E001": 0.88,

"E002": 0.92,

"E003": 0.45,

"E004": 0.44,

"E005": 0.30

}

}

4.2 Expected LLM Output (Correlation Only)

{

"situation_summary": "Five events form a single correlated situation centered on Agg-SW1 and its downstream path.",

"grouping": {

"core_events": ["E001","E002"],

"downstream_symptoms": ["E003","E004","E005"]

},

"why_grouped": [

"All events occur within a short time window (temporal correlation).",

"All devices are in the same site and zone (spatial correlation).",

"Edge and App devices are downstream of Agg-SW1 in topology (topological correlation).",

"Event semantics form a causal chain: utilization -> drops -> latency -> response time."

],

"relative_importance": [

{"event":"E002","score":0.92},

{"event":"E001","score":0.88},

{"event":"E003","score":0.45},

{"event":"E004","score":0.44},

{"event":"E005","score":0.30}

],

"confidence": "High"

}

No RCA, no remedies—only correlation structure and grouping.

5) Demo Workflow (Pictorial)

[ Ingest Data ]

|-- Events

|-- Metrics

|-- NetFlow

|-- Topology

|-- Changes

|

v

[ Preprocess & Enrich ]

|

v

[ Rule Correlation Layer ]

|-- Temporal edges

|-- Spatial edges

|-- Topological edges

|-- Causal/Rule edges

|

v

[ Correlation Graph ]

(Nodes: E001..E005, Edges: multi-type, weighted)

|

v

[ GNN Refinement (optional) ]

|-- Node influence scores

|-- Edge weight tuning

|

v

[ LLM Correlation Synthesis ]

|-- Inputs: graph + metrics + topology + scores

|-- Output: grouped situation + explanation

|

v

[ Final Correlated Situation ]

|-- One cluster

|-- Core vs downstream roles

|-- Confidence

6) What This Demo Proves (Correlation Only)

- Multiple heterogeneous signals converge into one situation

- Each correlation type contributes:

  - Temporal → time window coherence

  - Spatial → locality

  - Topological → dependency path

  - Causal/Rules → semantic chain

- GNN ranks importance

- LLM explains the grouping in human terms

- No RCA is performed—only correlation and grouping